SPECIFICATION OF CIVIL WORK for Minimum Construction Area — Plot Size 1000 Sqmtr

This document outlines the standard scope of civil construction work undertaken by Lakha's Group for a minimum completion area on a plot size of 1000 Sqmtr, along with applicable rates and payment terms. Plot & Coverage Summary PLOT SIZE; TOTAL COVERAGE. 1000 Sqmtr; 280 Sqmtr. Excavation - The contractor shall carry out excavation in soft soil up to a minimum of 2 feet, or as required for foundations as per the design of the structural engineer/architect. Ground Floor - Slab top height: 9–10 ft. from finish floor level. - Load-bearing wall structure with all necessary 2nd quality brick work, covering a completion area of 280 Sqmtr, including fixing of doors, windows and ventilators. - Inside/outside plaster (fine sand only) with complete flooring. - Plumbing and electrical conduit only. - Concrete in column/beam/slab of M20 grade, or as per structural drawings. Toilet / Bathroom - Internal plumbing fitting provision for seat, tap and waste-water outlet in the toilet. - Sewer & water line provided in 4 MPA/KG PVC pipe. Kitchen - Internal plumbing fitting provision in the kitchen with kitchen slab, one tap, and conduit only. Electrical Works - Installation of all PVC conduit pipes of required capacity, including internal flexible concealed PVC conduit for lighting, fans and socket outlets. - Only 2 switch boxes (8×3 or 4×3) and 1 light box provided in each bedroom/dining area. - Any luxurious fittings beyond the above will be provided at extra cost. - No accessories of any kind will be provided. Flooring & Terracing - CC flooring in grey cement for drawing room, bedroom, kitchen and verandah. - 3-inch brick ballast with 3-inch P.C.C. for ramp and driveway. Main Gate & Boundary Wall - Boundary wall provided on the front boundary, with fine-sand plaster throughout, total height of 7 ft. (or as required by local authority) from road level. - Side walls plastered, with boundary height of 5 ft. from DPC level. - DPC level: 1.5 ft. from road level. Main gate provided of approx. 100 kg. General Notes - All shuttering for R.C.C. work shall be of ply shuttering; all concrete work shall be done by mixer/vibrator with specified water-cement ratio and admixture as specified by the engineer. - All measurement shall be done on a slab-area basis. - All work shall follow the Architectural/Structural/Plumbing/Electrical drawings as specified by the engineer from time to time. - Scope of work does not include any interior jobs, furniture-making, landscaping, etc. - Design includes bedroom, toilet, kitchen and covered verandah only. - Any extra work as instructed by the site engineer will be charged extra. - Final payment shall be made after the satisfaction of the Architect/Employer. - This agreement is based on covered-area rate as per slab measurement; if the area increases, the rate will increase as per the site engineer's assessment. - This agreement is based on prevailing market rates — if the market rate increases, the cost of the agreement will increase accordingly. GST and any extra work will be chargeable additionally. Construction Cost Construction cost for making minimum completion area on a plot size of 1000 Sqmtr: Total Cost of Construction; Rs. 26,00,000 (Twenty-Six Lacs). Payment Plan PAYMENT STAGE; AMOUNT (RS.). Agreement Time; ________________. DPC Level; ________________. Soil Filling; ________________. Slab Level; ________________. Slab Casting; ________________. Plaster Work; ________________. Completion Certificate; ________________. Inclusions & Exclusions INCLUDED; EXTRA / EXCLUDED. Submersible Pump; Soil Filling. Map Approval; Water Meter & Electrical Connection Charges. Completion Certificate; Boring & Other Related Items. Time Duration: 90 days (subject to payments being made on schedule). Important Note - Charges for water meter, electrical connection, related bills, boring, and any other related items will be the responsibility of the owner. - Any item in addition to the details specified above will be charged extra. - In case the cost of any item specified in the contract is exceeded, the difference in cost will be charged to the owner. We assure you our best services and quality construction. For any clarification or doubt, please contact us at the number below. First Party — Lakha's Group; Second Party — Client/Owner.

LAKHA'S GROUP Contact: +91 63961 43510 ¦ Virat Raj Singh Lakha